FlowSplat: Transport-Constrained Gaussian Splatting for Joint Vessel Geometry and Flow Recovery from Dynamic X-ray Angiography

Target venue: CVPR 2027 main track. Plan Version 1, frozen on Sep 29, 2026. Strategy changes are allowed only at the decision gates.

# Goal

From a sparse sequence of dynamic X-ray projections (contrast filling a vessel), use 3D Gaussian Splatting to jointly recover (1) the 3D vessel lumen, (2) volumetric blood flow Q, and (3) FFR, with no pressure wire. Evaluate first on synthetic data with ground truth, then on a bifurcation, then on real angiography.

Personal target: a complete, submission-ready paper by Nov 10, 2026. Nov 11 to 16 is buffer and polish only.

## Key Dates

| Date (2026) | Event |
| --- | --- |
| Sep 30 | Plan starts (Step 0) |
| Oct 20 | Gate 1: Q recovered on single vessel (go / no-go for CVPR) |
| Oct 26 | ISBI 2027 deadline (other project, light week here) |
| Nov 2 | Gate 2: baselines, FFR and bifurcation status |
| Nov 9 | Gate 3: real-data status, experiments frozen |
| Nov 10 | CVPR paper registration (AOE, fixed) + my full draft complete |
| Nov 16 | CVPR full paper submission (AOE, fixed, no extension) |
| Nov 23 | CVPR supplementary material deadline (AOE) |

# Project Summary

Wire-free functional assessment of coronary stenosis (FFR) needs two things from angiography: accurate 3D lumen geometry and the actual blood flow. Clinical tools such as QFR reconstruct geometry from two views and estimate flow separately, either with a fixed empiric velocity (0.35 m/s) or by TIMI frame counting. Recent Gaussian splatting methods for dynamic DSA (TOGS, 4DRGS) reconstruct vessels efficiently, but model contrast change over time as a free, unconstrained network output and average the time axis away, discarding the flow information it carries. Physics-informed methods (PUNCH) recover velocity from contrast transport, but only along a 2D projected centerline of a single vessel, so they are biased by foreshortening and cannot give lumen area or FFR.

FlowSplat closes this gap by putting contrast-transport physics inside a Gaussian representation: the same Gaussians define the lumen area and carry the contrast bolus, so flow Q becomes a directly optimized, physically meaningful parameter. FFR then follows from (A, Q) through a reduced-order stenosis pressure-drop model.

# Contributions

- Transport-constrained Gaussians. Replace the free time network of 4DRGS with plug-flow contrast transport, so per-branch flow Q is read directly from the reconstruction.

- Joint 3D geometry and flow. Lumen area comes from the Gaussians (A = 2πσ²), removing the foreshortening bias of 2D methods (about 13% velocity underestimate at 30° out of plane).

- Bifurcation physics. Hard mass-conservation constraint Q_p = Q₁ + Q₂ and a soft Murray prior r_p³ = r₁³ + r₂³, going beyond PUNCH's single vessel.

- End-to-end FFR from the same Gaussians via a Young-Tsai pressure-drop model.

- Synthetic benchmark on real ImageCAS coronary geometry with ground-truth Q and FFR.

# Approach

## Forward model

Each Gaussian i has static geometry (position, rotation, scale) and sits at arc length s_i along its branch. For log-subtracted DSA, the rendered pixel uses R²-Gaussian's rectified X-ray rasterizer:

y(p, t) = Σᵢ μ_I · cᵢ(t) · Gᵢ²ᴰ(p)

Contrast concentration is not free. It follows plug-flow advection, justified by PUNCH's reported Péclet number of about 4000 (advection dominates, dispersion matters only at the bolus fronts):

cᵢ(t) = C_in(t − τᵢ),     τᵢ = ∫₀^{sᵢ} A(s) / Q ds,     A(s) = 2π σ(s)²

Unknowns: Gaussian geometry, one Q per branch, and a gamma-variate inlet curve C_in(t). The radius equivalence R = √2·σ comes from matching projected mass: μ₀·2πσ² = μ₀·πR².

## FFR

ΔP = (8πμL · A_n / A_s²) · V + (ρ K_t / 2) · (A_n / A_s − 1)² · V²,     FFR = (P_a − ΔP) / P_a

Worked example (motivation figure): 50% diameter stenosis (3.0 to 1.5 mm), L = 5 mm, μ = 3.5 mPa·s, ρ = 1060 kg/m³, K_t = 1.52, Q = 3 mL/s (V = 0.424 m/s), P_a = 90 mmHg.

- Viscous term 422 Pa, expansion term 1305 Pa, ΔP = 12.96 mmHg, FFR = 0.856.

- A 0.1 mm error in stenosis diameter (1.4 mm) gives ΔP = 18.2 mmHg, FFR = 0.798, crossing the 0.80 treatment threshold.

- A 30° foreshortening error in V (V measured as 0.866·V) gives ΔP = 10.1 mmHg, FFR = 0.888.

Conclusion: geometry and flow must be estimated jointly and in 3D.

## Bifurcation

Hard constraint Q_p = Q₁ + Q₂. Soft prior r_p³ = r₁³ + r₂³. Example: r_p = 1.5 mm, r₁ = 1.25 mm gives r₂ = 1.125 mm and flow split Q₁/Q_p = 1.953 / 3.375 = 0.58.

# Related Work / References

Anchor papers (what we extend):

- Liu, Zha, Zhao, Li, Cui. "4DRGS: 4D Radiative Gaussian Splatting for Efficient 3D Vessel Reconstruction from Sparse-View Dynamic DSA Images." MICCAI 2025. arXiv:2412.12919.

(method base: setting, 30/40/50/60-view protocol, main baseline. Its Discussion names coronary data, bi-plane DSA and cardiac motion as future work)

- Thakur et al. "PUNCH: Physics-informed Uncertainty-aware Network for Coronary Hemodynamics." arXiv:2601.17192, 2026.

(physics base: 1D advection-diffusion, flow ranges, Péclet argument. Its Limitations name bifurcations, vessel overlap and multi-branch geometry as future work)

Gaussian splatting for X-ray (representation and rendering):

- Zha et al. "R²-Gaussian: Rectifying Radiative Gaussian Splatting for Tomographic Reconstruction." NeurIPS 2024. arXiv:2405.20693.

(X-ray rasterizer and voxelization we build on)

- Cai et al. "Radiative Gaussian Splatting for Efficient X-ray Novel View Synthesis" (X-Gaussian). ECCV 2024.

- Zhang et al. "TOGS: Gaussian Splatting with Temporal Opacity Offset for Real-Time 4D DSA Rendering." IEEE JBHI 2025. arXiv:2403.19586.

(notes that time-density curves allow flow quantification, but does not do it)

- "DSA-SRGS: Super-Resolution Gaussian Splatting for Dynamic Sparse-View DSA Reconstruction." arXiv:2603.04770, 2026.

- "3DGR-CAR: Coronary Artery Reconstruction from Ultra-sparse 2D X-Ray Views with a 3D Gaussians Representation." MICCAI 2024.

- Maas et al. "NerT-CA: Efficient Dynamic Reconstruction from Sparse-View X-ray Coronary Angiography." RIME/GRAIL 2025.

- "PINGS-X: Physics-Informed Normalized Gaussian Splatting with Axes Alignment for Efficient Super-Resolution of 4D Flow MRI." AAAI 2026. arXiv:2511.11048.

(Gaussians + hemodynamics, but MRI only)

Flow from angiography (closest non-Gaussian work):

- "Physics-Informed Learning for Time-Resolved Angiographic Contrast Agent Concentration Reconstruction." arXiv:2403.01993, 2024.

- "Spatiotemporally Constrained 3D Reconstruction from Biplanar Digital Subtraction Angiography." PMC12350070.

- "Blood Flow and Velocity Estimation Based on Vessel Transit Time by Combining 2D and 3D X-Ray Angiography." MICCAI 2006.

(classic two-stage baseline: geometry first, then velocity from time-density curves)

Clinical baselines and physics models:

- "Overview of Quantitative Flow Ratio and Optical Flow Ratio in the Assessment of Intermediate Coronary Lesions." PMC11664755.

(fQFR with 0.35 m/s, cQFR with TIMI frame count)

- FAVOR II Europe-Japan study: QFR vs pressure-wire FFR. J Am Heart Assoc, 2018.

- Young, Tsai. "Flow characteristics in models of arterial stenoses." J Biomech, 1973. [verify exact citation]

- Murray. "The physiological principle of minimum work." PNAS, 1926. [verify exact citation]

# Data & Tools

- Biguri et al. "TIGRE: a MATLAB-GPU toolbox for CBCT image reconstruction." Biomedical Physics & Engineering Express, 2016.

- Zeng et al. "ImageCAS: A large-scale dataset and benchmark for coronary artery segmentation." Computerized Medical Imaging and Graphics, 2023.

- ASOCA: Automated Segmentation Of Coronary Arteries challenge dataset (60 CCTA).

- "From Centerlines to Hemodynamics: Anisotropic RBF Decoders for Coronary Arteries." 2026. Dataset: huggingface.co/datasets/angioinsight/single-vessel-flow.

- Jiménez-Partinen et al. "CADICA: a new dataset for coronary artery disease detection by using invasive coronary angiography." Expert Systems, 2024. (668 videos, 42 patients, 10 fps)

- "AngioCAD: A public x-ray angiography dataset and an adaptive fusion framework for stenosis detection." 2026 (413 patients). DIAS (arXiv:2306.12153) and DSCA (arXiv:2406.00341): public cerebral DSA sequences.

# Dataset

No public dataset pairs multi-view dynamic angiography with ground-truth flow or FFR, so data follows three layers:

- Synthetic with ground truth (main results). LAD segments from ImageCAS (centerline + radius from expert masks, reusing my existing extraction code), plus synthetic stenoses (30 to 70% diameter). Flow sampled from physiological ranges: rest velocity 10 to 20 cm/s (PUNCH), hyperemia up to about 0.4 m/s. Bolus simulated with 1D advection-diffusion on a fine grid with dispersion and noise, deliberately richer than the plug-flow model we fit, to avoid an inverse crime. Rotational DSA rendered with TIGRE following 4DRGS (133 frames over 198°, each frame at its own time), subsampled to 30 / 60 / 133 views.

- Bifurcation set. Y-bifurcations (LAD + diagonal) from ImageCAS with a Murray-consistent flow split.

- Real data. CADICA (monoplane, qualitative only). 4DRGS and PUNCH clinical data only if obtained by request.

Timing check before generating data: 198° over about 4.4 s with 30 views means one view every 0.147 s. A 50 mm segment at 0.42 m/s is crossed in 0.12 s, less than one view interval. So start with rest flow (0.15 m/s, 0.33 s transit), full LAD length (about 100 mm), and sweep the number of views.

# Evaluation Plan

| Category | Metric |
| --- | --- |
| Flow | Relative error of Q (headline). Flow split error at bifurcation |
| Function | FFR MAE; accuracy / sensitivity / specificity at FFR 0.80 |
| Geometry | Stenosis diameter error (mm), Chamfer distance, lumen radius error |
| Rendering | PSNR / SSIM on held-out views (comparable to 4DRGS) |
| Robustness | Sweeps: number of views (30/60/133), noise, Péclet, flow speed, foreshortening angle |

Baselines:

- 4DRGS (free-time Gaussians) + post-hoc velocity from time-density curves.

- PUNCH-style 1D advection fit on the 2D projected kymograph (exposes foreshortening bias).

- R²-Gaussian static geometry + fixed 0.35 m/s flow (fQFR analog).

- Oracle: ground-truth geometry and Q through Young-Tsai (separates reduced-order model error from reconstruction error).

# Risks & Scope

- Geometry / flow ambiguity: a narrower vessel with lower Q can explain the same image intensity. Mitigation: bolus arrival timing depends on Q, not intensity; staged optimization (geometry first, then Q, then joint).

- Temporal sampling: hyperemic transit can be shorter than the interval between sparse views. Mitigation: rest flow first; view sweep reported as a result.

- Inverse crime: simulating and fitting with the same model. Mitigation: richer simulator than the fitting model; report Péclet robustness.

- Sim-to-real gap: all quantitative results are synthetic. Mitigation: real-data experiment (Step 6) and an honest limitations section.

- Compute: no CUDA on local M4; dependency on Kaggle / cloud GPU for the rasterizer.

- Timeline: overlaps with ISBI (Oct 26).

In scope: static geometry, steady (non-pulsatile) flow, single vessel with one stenosis, one bifurcation, synthetic data with ground truth, qualitative real-data test.

Out of scope (future work): cardiac and respiratory motion, pulsatile flow, full coronary trees, quantitative validation against invasive FFR on patients.

# Ethical Implications

This project uses public, de-identified datasets (ImageCAS, CADICA) and synthetic simulations, and produces no clinical or diagnostic output. Reconstructed flow and FFR are evaluated against simulated ground truth in a research setting only. Because FFR directly drives stenting decisions, a wrong estimate carries real patient risk: any future clinical use would require validation against invasive FFR on real patients, calibrated uncertainty, and human-in-the-loop oversight. ImageCAS and CADICA each come from limited sites and scanners and may not represent the full range of anatomy and equipment. This project is a feasibility study of the joint geometry and flow reconstruction problem, not a clinical tool.

# Work Steps Breakdown

## Step 0: Setup

### 0.1 GPU environment and code base [SMALL] [Deps: None] _ Sep 30 to Oct 4

- Run R²-Gaussian (X-ray rasterizer) on a CUDA GPU (Kaggle or cloud), reproduce one static reconstruction.

- Check whether 4DRGS code is public; if yes, run it on one case.

- Confirm TIGRE on the same GPU environment (already working in the previous project).

- Create / update OpenReview profile with institutional email (non-institutional profiles can take up to 2 weeks of moderation).

- Email 4DRGS (Zhiming Cui, cuizhm@shanghaitech.edu.cn) and PUNCH (Sukirt Thakur, sukirt@angioinsight.com) authors asking for data access. Do not share method details. Download CADICA (data.mendeley.com/datasets/p9bpx9ctcv/2) to Google Drive.

DoD (Definition of Done): Static reconstruction of one ImageCAS vessel with R²-Gaussian renders correctly; OpenReview profile active; data-request emails sent.

How I plan to do it: GPU notebook for all rasterization; M4 for scripting and analysis only.

Repo: New repo src/flowsplat/; environment notes in SETUP.md.

Fallback: If the CUDA rasterizer will not build: another cloud GPU. If nothing works by Oct 4: plain PyTorch Gaussian splatter (slower, no custom CUDA). Do not change the project.

## Step 1: Synthetic Data Generation

### 1.1 Geometry: centerlines, stenoses, flow and FFR ground truth [MEDIUM] [Deps: 0.1] _ Oct 5 to Oct 8

- Extract LAD centerline + radius from ImageCAS (reuse centerline.py).

- Insert synthetic stenoses (30 to 70% diameter); record ground-truth geometry.

- Sample Q from physiological ranges; compute ground-truth FFR with Young-Tsai.

- Run the timing check (Dataset section) before generating the full set.

DoD (Definition of Done): About 200 cases, each with geometry, Q and FFR saved; ranges checked (rest 10 to 20 cm/s).

How I plan to do it: Runs on my M4 locally, CPU only.

Repo: src/flowsplat/data_gen.py

Fallback: If ImageCAS extraction causes problems: procedural vessels with realistic ranges; ImageCAS moves to supplementary.

### 1.2 Bolus simulation and TIGRE rendering [MEDIUM] [Deps: 1.1] _ Oct 8 to Oct 11

- Simulate bolus with 1D advection-diffusion on a fine grid, with dispersion and noise.

- Map concentration onto the 3D lumen per time step.

- Render rotational DSA with TIGRE (133 frames over 198°, each at its own time); store 30 / 60 / 133-view subsets.

- Case-level split: train / val / test.

DoD (Definition of Done): All cases rendered; 5 cases spot-checked visually (bolus visibly advancing along the vessel); fixed split lists saved.

How I plan to do it: Bolus simulation on M4 (1D, CPU); projection on GPU with TIGRE.

Repo: src/flowsplat/bolus_sim.py, render_tigre.py

Fallback: If 3D bolus mapping is too slow: pre-compute concentration on the centerline and splat it into the volume at render time.

## Step 2: FlowSplat Core (single vessel)

### 2.1 Geometry init and tube-aligned Gaussians [MEDIUM] [Deps: 1.2] _ Oct 12 to Oct 14

- Static R²-Gaussian on time-averaged views (as in 4DRGS voxelization).

- Map Gaussians to centerline arc length s_i; tube-alignment regularizer (major axis along tangent).

DoD (Definition of Done): Lumen radius error below 0.2 mm on 5 validation cases.

How I plan to do it: Full runs on GPU.

Repo: src/flowsplat/model.py

Fallback: If Gaussians do not align with the vessel: initialize directly from the ground-truth-free FDK volume skeleton.

### 2.2 Transport layer and joint optimization [LARGE] [Deps: 2.1] _ Oct 14 to Oct 19

- Parameters Q and gamma-variate C_in; compute τ(s) and time-dependent concentration per Gaussian.

- Stage A: geometry frozen, fit Q. Stage B: joint fine-tune with L1 + D-SSIM image loss.

- First overfit one case, then 20-case validation.

DoD (Definition of Done): On 20 validation cases: Q relative error below 10% and stenosis diameter error below 0.2 mm (30 or 60 views).

How I plan to do it: Prototype the transport layer on M4 (small PyTorch); full runs on GPU.

Repo: src/flowsplat/transport.py, train.py

Fallback: Q does not converge: grid search over Q, then fine-tune. Geometry/flow ambiguity: keep geometry frozen longer, add arrival-time loss. 30 views insufficient: use 60 and report error vs views as a result.

## Step 3: Robustness and FFR

### 3.1 Sweeps and FFR [MEDIUM] [Deps: 2.2] _ Oct 20 to Oct 23 (ISBI week, background runs only)

- Queue sweeps: views (30/60/133), noise, Péclet, flow speed, foreshortening angle.

- Add FFR (Young-Tsai on recovered A(s), Q) and the oracle comparison.

DoD (Definition of Done): Sweep tables saved; FFR MAE and accuracy at 0.80 on validation set.

How I plan to do it: Launch GPU jobs early in the week; check results after ISBI submission.

Repo: src/flowsplat/ffr.py, sweeps.py, metrics.py

Fallback: If FFR error is large: decompose with the oracle; center the paper on Q and present FFR as an application.

## Step 4: Baselines

### 4.1 Baseline implementation and main table [MEDIUM] [Deps: 2.2] _ Oct 23 to Oct 26

- 4DRGS + post-hoc time-density-curve velocity.

- PUNCH-style 1D fit on the 2D projected kymograph.

- R²-Gaussian geometry + fixed 0.35 m/s.

- Oracle.

DoD (Definition of Done): One results table (Q error, FFR MAE, FFR accuracy, diameter error) on the held-out test set for all methods.

How I plan to do it: Reuse Step 2 components; baselines are mostly subsets of the model.

Repo: src/flowsplat/baselines/

Fallback: If FlowSplat does not beat baseline 1 on average: show the advantage in hard regimes (few views, fast flow, strong foreshortening).

## Step 5: Bifurcation (after single-vessel results are complete)

### 5.1 Y-bifurcation with continuity and Murray prior [MEDIUM] [Deps: 2.2] _ Oct 27 to Nov 1

- Y-bifurcation cases from ImageCAS (LAD + diagonal).

- Per-branch Q with hard constraint Q_p = Q₁ + Q₂ and soft Murray prior.

- Ablation: with vs without continuity / Murray.

DoD (Definition of Done): Flow split error below 10% on bifurcation test cases; ablation table.

How I plan to do it: Same training code with a branch index per Gaussian.

Repo: src/flowsplat/tree.py

Fallback: If it does not work by Nov 2: submit with single vessel only; bifurcation becomes future work. The paper remains valid.

## Step 6: Real Data Test (after synthetic results are complete)

### 6.1 Real angiography [MEDIUM] [Deps: 2.2, 4.1] _ Nov 2 to Nov 6

- If 4DRGS or PUNCH data arrived: run the full method and report quantitatively.

- Otherwise, CADICA (monoplane): select 5 to 10 LAD videos with clear contrast, fit the transport model (bolus arrival time) on the real vessel, compare velocity with TIMI frame count, and check the physiological range (rest 10 to 20 cm/s).

DoD (Definition of Done): One real-data figure plus a short table showing the method runs and gives plausible values on real angiography.

How I plan to do it: Preprocessing (segmentation, centerline) on M4; fitting on GPU.

Repo: src/flowsplat/real/

Fallback: If it fails on real data: synthetic-only paper with an honest limitations section; decide at Gate 3.

## Step 7: Writing and Submission

### 7.1 Paper draft [LARGE] [Deps: all] _ Oct 27 to Nov 10

- Oct 27 to Nov 2: introduction, related work, method (do not depend on final numbers).

- Nov 3 to Nov 9: experiments, figures (FFR motivation example, bolus rendering, results table, sweeps, real-data figure), limitations.

- Nov 10: register paper on OpenReview (title, abstract, authors); full draft complete.

DoD (Definition of Done): Complete CVPR-format draft on Nov 10.

How I plan to do it: CVPR LaTeX template; all figures generated by scripts from saved results.

Repo: paper/ folder in repo

Fallback: If Gate 3 decides against CVPR: same draft is retargeted to CVPR 2027 workshops or MICCAI 2027.

### 7.2 Polish, submission and supplementary [SMALL] [Deps: 7.1] _ Nov 11 to Nov 23

- Nov 11 to 16: polish, proofread, final submission.

- Nov 17 to 23: supplementary (bolus video, extra sweeps).

DoD (Definition of Done): Submitted PDF on Nov 16; supplementary on Nov 23.

How I plan to do it: No new experiments in this window.

Repo: paper/

Fallback: None needed; this is buffer.

# Weekly Schedule

| Week | Dates (2026) | Question of the week | Outputs | Gate |
| --- | --- | --- | --- | --- |
| W1 | Sep 30 to Oct 4 | Does the X-ray Gaussian rasterizer run on my GPU setup? | Static reconstruction, OpenReview profile, data-request emails |  |
| W2 | Oct 5 to Oct 11 | Can I generate realistic bolus DSA with ground truth? | ~200 synthetic cases, timing check, split |  |
| W3 | Oct 12 to Oct 19 | Does Q come back on a single vessel? | FlowSplat core, 20-case validation numbers | Gate 1: Oct 20 |
| W4 | Oct 20 to Oct 26 | Are results robust, and do we beat baselines? | Sweeps, FFR, baseline table (buffer week if Step 2 slips) |  |
| W5 | Oct 27 to Nov 1 | Does the bifurcation work? | Bifurcation results, method section drafted | Gate 2: Nov 2 |
| W6 | Nov 2 to Nov 9 | Does it run on real angiography? Is the paper complete? | Real-data figure, all figures, full draft | Gate 3: Nov 9 |
| W7 | Nov 10 to Nov 16 | Is the paper ready to submit? | Nov 10 registration + full draft; Nov 16 submission |  |
| W8 | Nov 17 to Nov 23 | Supplementary | Video, extra results |  |

# Decision Gates (the only moments strategy can change)

| Gate / date | Condition | Pre-committed decision |
| --- | --- | --- |
| Oct 4 (setup) | CUDA rasterizer does not run | Plain PyTorch splatter; do not change the project |
| Gate 1: Oct 20 | Q error below 10% on single vessel? | Yes: continue to CVPR. No: drop CVPR main track, retarget CVPR 2027 workshops or MICCAI 2027 (about March), same plan |
| Gate 2: Nov 2 | Bifurcation works? | No: submit single vessel only; bifurcation is future work |
| Gate 2: Nov 2 | Beat baselines on average? | No: present hard-regime advantage (few views, fast flow, foreshortening) |
| Gate 3: Nov 9 | Any real-data result? | No: synthetic + honest limitations; submit if Gates 1 and 2 are strong, otherwise retarget |

# Focus Rules (how I stay on this plan)

- The plan is frozen. This document is Version 1 (Sep 29). Strategy changes only at the decision gates, never in the middle of a week.

- Decisions are pre-committed. Every step has a DoD and a fallback. When a result is bad, I do not look for a new idea: I open this document and execute that step's fallback.

- Idea parking lot. Any new idea during the work (cardiac motion, diffusion priors, other methods) goes into the Parking Lot table only, and is not opened until after Nov 16.

- Time box for debugging. Maximum 2 days per problem. If not solved, apply the fallback, never a new branch.

- One question per week. Each week has one question (Weekly Schedule). Anything that does not help answer it goes to the parking lot.

- Log negative results, do not run from them. If something fails, write why in the Negative Results Log. Diagnosed failures become ablations or limitations in the paper.

Golden rule: no strategic decision in the middle of a week. Only at a gate.

# Idea Parking Lot (do not open before Nov 16)

| Date | Idea | Why it came up |
| --- | --- | --- |
| Sep 29 | Cardiac motion + contrast filling disentanglement | Needed for real coronary cine (4DRGS limitation) |
| Sep 29 | TreeSplat: topology-native graph Gaussians for full trees | Connectivity problem from previous project |

# Negative Results Log

| Date | Step | What failed | Diagnosis | Action (fallback used) |
| --- | --- | --- | --- | --- |

# Work Process Log

Weekly entries: what was done, numbers obtained, and whether the week's question was answered.

W1 (Sep 30 to Oct 4):

W2 (Oct 5 to Oct 11):

W3 (Oct 12 to Oct 19):

W4 (Oct 20 to Oct 26):

W5 (Oct 27 to Nov 1):

W6 (Nov 2 to Nov 9):

W7 (Nov 10 to Nov 16):